Παράρτημα

Σχέση σχημάτων χρέωσης με φράγματα του ισοδυνάμου εύρους ζώνης

Αυτό που επιχειρούμε μέσω των συναρτήσεων χρέωσης είναι να φράξουμε το ισοδύναμο εύρος ζώνης αs,t με την βοήθεια γραμμικών συναρτήσεων fx. Τότε,

αs,t=1stlogEesX0,t≤1stlogEfsX0,tÛ			EesX0,t≤EfsX0,t

Σχήμα 1: Οι συναρτήσεις χρέωσης ως άνω φράγματα του ισοδυνάμου εύρους ζώνης

Ας εξετάσουμε την περίπτωση της “Απλής” φόρμουλας, δηλαδή να θεωρήσουμε την απλούστερη συνάρτηση παρεμβολής (βλέπε Σχήμα 1)

f1x=xshtesht−1+1.

Τότε

Ef1sX0,t=EX0,tthesht−1+1

και ασφαλώς

αs,t≤1stEf1sX0,t=1stmhesht−1+1	(Π.1)

Παρατηρείστε ότι το δεξιό μέρος της ισότητας στην (Π.1), είναι ίσο με την προσέγγιση που χρησιμοποιεί η “Απλή” φόρμουλα χρέωσης για το ισοδύναμο εύρος ζώνης μιας πηγής, θεωρώντας την ON/OFF (βλέπε Σχέση 2.6).

Στην περίπτωση των δύο βαθμίδων χρησιμοποιούμε την συνάρτηση

f2x=1+xskteskt−1⋅1x<skt         +eskt+x−sktshtesht−eskt1x>skt

και αποδεικνύεται (βλέπε κατωτέρω) ότι

αs,t≤1stEf2sX0,t.			(Π.2)

Άρα, και στις δύο περιπτώσεις (“Tax-Band” με L=1 και “Απλή” φόρμουλα), χρησιμοποιείται η ίδια συνάρτηση-φράγμα για χρέωση. Συνεπώς, οι συναρτήσεις f1x, f2x με τις οποίες κάνουμε παρεμβολή, δεν είναι άλλες από αυτές που χρησιμοποιούν τα δύο σχήματα χρέωσης που έχουμε δει έως τώρα. Έτσι το “Tax-Band” Σχήμα είναι μια λογική γενίκευση της “Απλής” φόρμουλας, που προσεγγίζει ακριβέστερα την καμπύλη αs,t. Για δεδομένα σημεία παρεμβολής (αντίστοιχα βαθμίδες χρέωσης L=1,2) η χρέωση (ανά μονάδα χρόνου) είναι κατά μέση τιμή μεγαλύτερη από το ισοδύναμο εύρος ζώνης (ασυμπτωτικά ίσο, για μεγάλες τιμές των L και T).

Από τα παραπάνω συμπεραίνουμε ότι στην περίπτωση υποθετικής πηγής, για την  οποία Xt,t+tÎ0,kt,ht, η ανισότητα (Π.2) γίνεται ισότητα. Αντίστοιχα για την (Π.1) αν Xt,t+tÎ0,ht. Έτσι η ON/OFF προσέγγιση κίνησης είναι η χειρότερη όσο αφορά το ισοδύναμο εύρος ζώνης. Η προσέγγιση που επιχειρεί το “Tax-Band” με κίνηση τριών επιπέδων είναι ακριβέστερη.

Το “Tax-Band” ως άνω φράγμα του ισοδυνάμου εύρους ζώνης

Θα αποδείξουμε την σχέση ενός άνω φράγματος για το ισοδύναμου εύρους ζώνης αZ, στο οποίο βασίζεται η χρέωση του  “Tax-Band” σχήματος.

Έστω πηγή με μέγιστο ρυθμό κυκλοφορίας h, και k το μεσαίο φράγμα. Αν T η συνολική διάρκεια της κλήσης, υπολογίζουμε τα T/t υποδιαστήματα διάρκειας t. Έστω ότι το ht έχει μεγάλη και ακέραια τιμή. Συμβολίζουμε με Nu τον αριθμό των διαστημάτων στα οποία η κυκλοφορία της πηγής ισούται με u, όπου u ακέραιος μεταξύ του 0 και ht (αυτός είναι ο μέγιστος αριθμός μονάδων κυκλοφορίας που μπορεί να παραχθεί στη διάρκεια ενός υποδιαστήματος). Τότε

αZ=1stlogZ

»1stlog1Ttu=0htNuesu 	      (Π.3)

=1stlog1Ttu=0ktNuesu+1Ttu=kt+1htNuesu

1stlog1Ttu=0ktNukt−u+uesktkt+1Ttu=kt+1htNuht−ueskt+u−kteshtht−kt(P.4)           =1stlogu=0ktNukt+uNueskt−1kT+u=kt+1htNuhteskt−ktesht+uNuesht−eskth−kT

=T1T+V1Teskt−1k+T2Theskt−keshth−k+V2Tesht−eskth−k

=1stlogF1+eskt−1kM1+heskt−keshth−kF2+esht−eskth−kM2

=α(F1,F2,M1,M2),

όπου Fi=TiT και Mi=ViT. Παρατηρείστε ότι στην περίπτωση παρατεταμένων μετρήσεων, π.χ. Tt->¥, η (Π.3) ισχύει σαν ισότητα, με αποτέλεσμα η ανισότητα (Π.4) να εξασφαλίζει ένα συντηρητικό εκτιμητή του ισοδύναμο εύρος ζώνης, τον

α(Z)=limT/t→¥α(F1,F2,M1,M2).

Είδαμε ότι η “Απλή” Φόρμουλα χρέωσης χρεώνει βέλτιστα όταν ο χρήστης προβλέψει σωστά το μέσο ρυθμό της κίνησης του, δεδομένου ότι η (3.3) είναι εφαπτόμενη της BM στο σημείο m=M. Αντίστοιχα η συνάρτηση (3.7) είναι το εφαπτόμενο υπερεπίπεδο της επιφάνειας α(F1,F2,M1,M2) στα σημεία F1=f1, F2=f2, M1=m1 και M2=m2 (έτσι υπολογίζονται και οι σταθερές της φόρμουλας). Όταν ο χρήστης προβλέψει σωστά, χρεώνεται και με αυτό το σχήμα βέλτιστα.

Βιβλιογραφία

[1] The ATM Forum (1996). ATM Forum Traffic Management Specification: Version 4.0.

[2] Alles, A. (1995). ATM Internetworking. Technical report, Cisco Systems.

URL: http://www.cisco.com/warp/public/614/12.html.

[3] Anick, D., D. Mitra, and M. M. Sondhi (Oct. 1982). Stochastic theory of a data-handling system with multiple sources. The Bell System Technical Journal, 61(8):1871-1894.

[4] Bailey, J. P. and L. Mcknight (1996). Internet Economics. MIT Press, Massachusetts.

[5] Bucklew, J. A. (1990). Large Deviation Techniques in Decision, Simulation and Estimation. John Wiley & Sons, New York.

[6] Beran. J., S. Sherman, M. S. Taqqu, and W. Willinger (1995). Variable-bit-rate video traffic and long-range dependence. IEEE Transactions on Communications, 43:1566-1579.

[7] Botvick, D. D. and N. Duffield (1995). Large deviations, the shape of the loss curve, and economies of scale in large multiplexers. Queueing systems, 20: 293-320.

[8] Chatfield, C. (1975). The Analysis of Time Series: Theory and Practice. Chapman and Hall.

[9] CASHMAN. Charging and Accounting Schemes in Multi-service ATM Networks (Ca$hMAN), ACTS Project (AC-039). URL: http://www.isoft.intranet.ge/cashman/.

[10] Charmantzis, F. C., C. Courcoubetis, V. A. Siris, and G. D. Stamoulis (1996). A Comparative Study of Usage-Based Charging Schemes. In Proceedings of the Colloquium on Charging For ATM, IEE, London.

[11] Courcoubetis, C. and R. Weber (1995). Effective bandwidths for stationary sources. Prob. Eng. Inf. Sci., 9, 285-296.

[12] Courcoubetis, C., G. Kesidis, A. Ridder, J. Walrand, and R. Weber (1995). Admission control and routing in ATM networks using inferences from measured buffer occupancy. IEEE Transactions on Communications, 43, 1778-1784.

[13] Courcoubetis, C., G. Fouskas, and R. Weber (1995). An on-line estimation procedure for cell-loss probabilities in ATM links. In Proceedings of the 3rd IFIP Workshop on Performance Modelling and Evaluation of ATM Networks.

[14] Courcoubetis, C., V. A. Siris, G. D. Stamoulis, and Y. Markopoulos (1995). Charging issues for services in broadband networks. In Proceedings of the OECD  Conference on The Economics of the Information Society.

[15] Courcoubetis, C. and R. Weber (1996). Buffer overflow asymptotics for a buffer handling many traffic sources. Journal of Applied Probability, 33.

[16] Courcoubetis, C., F. P. Kelly, and R. Weber (1997). Measurement-based Charging in Communications Networks. In preparation.

[17] Courcoubetis, C., F. P. Kelly, V. A. Siris, G. D. Stamoulis, and R. Weber (1997). ABR pricing experiments in real network. International Conference on Telecommunications-ICT97, Melbourne, Australia.

[18] Draft International Standard: ISO/IEC DIS 13818-2 (1993). Generic Coding of Moving Pictures and Associated Audio Part 2, Video.

[19] Deering, S. (1995). Internet protocol version 6 (ipv6) specification. RFC 1883.

URL: http://www.globecom.net/(nobg,sv)/ietf/rfc/rfc1883.shtml

[20] de Veciana, G. and J. Walrand (1995). Effective bandwidths: call adission, traffic policing and filtering for ATM networks. Queueing Systems, 20, 37-59.

[21] de Veciana, G. and R. Baldick (Jan. 1995). Resource allocation in multi-service networks via pricing. Technical Report, Univ. Of Texas at Austin.

[22] Edell, R., N. McKeown, and P. P. Varaiya (Sept. 1995). Billing users and pricing for TCP. IEEE J. Selected Areas in Communications, 13(7):1162-1175.

[23] Frost, V. S. and B. Melamed (March 1994). Traffic modeling for telecommunications networks. IEEE Communications Magazine, 32(3):70-81.

[24] Garett, M. W. and  W. Willinger (Aug. 1994). Analysis, modeling and generation of self-similar VBR video traffic. In Proceedings of the  ACM SIGCOMM 94, London, UK, 269-280.

[25] Gibbens, R. J. (1996). Traffic characterisation and effective bandwidths for broadband network traces. In F. P. Kelly, S. Zachary and I. Ziedins, editors, Stochastic Networks: Theory and Applications. Oxford University Press, Oxford.

[26] Gibbens, R. J. and P. J. Hunt (1991). Effective bandwidths for the multi-type UAS channel. Queueing Systems, 9(1):17-28.

[27] Gong, K. L. (1994). Berkeley MPEG-1 Video Encoder, User’s Guide. University of California, Berkeley, Computer Science Division-EECS.

[28] Habib, I. W. and T. N. Saadawi (July 1992). Multimedia traffic characteristics in broadband networks. IEEE Communications Magazine, 48-54.

[29] Heyman, D. P., A. Tabatabai, and T. V. Laksman (March 1992). Statistical analysis and simulation study of video teleconference traffic in ATM networks. IEEE Transactions on Circuits and Systems for Video Technology, 29(1):49-59.

[30] Hui, J. Y. (1988). Resource allocation for broadband networks. IEEE J.  Selected Areas in Communications. 6(9):1598-1608.

[31] International Standard: ISO/IEC IS 11172-2 (1993). Coding of Moving Pictures and associated Audio for Digital Storage Media up to 1.5 Mbit/s Part 2, Video.

[32] ITU Recommendation I.121 (1992). Broadband Aspects of ISDN. Geneva.

[33] ITU Recommendation I.371 (1994). Traffic control and congestion control in B-ISDN. Geneva.

[34] Kleinrock, L. (1975). Queueing Systems. John Wiley & Sons, New York.

[35] Knuth, D. (1981). Seminumerical Algorithms, vol. 2 of The Art of Computer Programming. Addison-Wesley

[36] Kelly, F. P. (1991). Effective bandwidths at multi-class queues. Queueing Systems, 9:5-16.

[37] Kelly, F. P. (1994). On tariffs, policing and admission control of multiservice networks. Operations Research Letters, 15:1-9.

[38] Kelly, F. P. (1996). Notes on effective bandwidths. In F. P. Kelly, S. Zachary and I. Ziedins, editors, Stochastic Networks: Theory and Applications, pp. 141-168. Oxford University Press, Oxford.

[39] Kesidis, G., J. Walrand, and C. S. Chang (Aug. 1993). Effective bandwidths for multiclass Markov fluids and other ATM sources. IEEE/ACM Transactions on Networking, 1:424-428.

[40] Le Gall, D. (April 1991). MPEG: A video compression standard for multimedia applications. Communications of the ACM, 34(4):46-58.

[41] Low, S. H. and P. P. Varayia (Oct. 1993). A new approach to service provisioning in ATM networks. IEEE/ACM Trans. on Networking, 1(3):547-553.

[42] Mackie-Mason, J. K. and H. R. Varian (1994).  Pricing the Internet. Public Access to the Internet, B. Kahin, and J. Keller, Eds. Englewood Cliffs, New Jersey: Prentice-Hall.

[43] Mitchell, B. M. And I. Vogelsang (1991). Telecommunications Pricing Theory and Practice. Cambridge University Press.

[44] Mackie-Mason, J. K. and H. R. Varian (1993). Some economics of the Internet. Technical Report, Univ. Of Michigan.

[45] Maglaris, B., D. Atastassiou, P. Sen, G. Karlsson, and J. D. Robbins (July 1988). Performance models of statistical multiplexing in packet video communications. IEEE Transactions on communications, 36(7):834-843.

[46] Murphy, J., L. Murphy, and E. C. Posner (1994). Distributed pricing for embedded ATM networks. ITC 94, ed. J. Labetoulle and J. W. Roberts.

[47] Nikolaidis, I. and Ian F. Akyildiz. An overview of source characterization in ATM networks. To appear as chapter in the upcoming book State-of-the-art in Performance Modeling and Simulation: Networks, E.Gelenbe, J. Warland, K. Bagchi, G.Zobrist,  editors. Gordon & Breach Publishing Group.

[48] Nomura, M., T. Fujii, and N. Ohta (June 1989). Basic characteristics of variable rate video coding in ATM environment. IEEE J. Selected Areas Commun., 7(5):752-760.

[49] Norros, I. (1994). A storage model with self-similar input. Queueing Systems, 16:387-396.

[50] Papoulis, A. (1990). Probability, Random Variables, and Stochastic Processes. McGraw-Hill, New York, third edition.

[51] Press, W., S. Teukolsky, W. Vetterling, and B. Flannery (1994). Numerical Recipes in C. Cambridge University Press.

[52] Sen, P., B. Maglaris, N. Rikli, and D. Anastassiou (June 1989). Models for packet switching of Variable-Bit-Rate video sources. IEEE J. Selected Areas Commun., 7(5):865-869.

[53] Simonian, A. and J. Guilbert (1995). Large deviations approximation for fluid queues fed by a large number of on-off sources. IEEE J. Selected Areas Commun., 13, 1017-1027.

[54] Siu, Κ. and R. Jain (1995). A Brief Overview of ATM: Protocol Layers, LAN Emulation, and Traffic Management. Computer Communications Review (ACM SIGCOMM).

[55] Stamoulis, G. D., M. E. Anagnostou, and A. D. Georgantas (1994). Traffic source models for ATM networks: a survey. Computer Communications, 17, 428-438.

[56] Varian, H. R. (1992). Microeconomic Analysis. W. W. Norton, New York.

[57] Varian, H. R. (1993). Intermediate Microeconomics: A Modern Approach. W. W. Norton, New York.

[58] Warland, J. and P. P. Varaiya (1996). High-Performance Communication Networks. Morgan Kaufmann, San Francisco.

[59] Weiss, A. (1986). A new technique for analysing large traffic techniques. Adv. Appl. Prob., 18, 506-532.

[60] Weiss, A. (1995). An introduction to Large Deviations for communications networks. IEEE JSAC, 13(6):938-952.

[61] Whitt, W. (1993). Tail probabilities with statistical multiplexing and effective bandwidths in multi-class queues. Telecommunication Systems, 2, 17-107.

[62] Willinger, W., M. S. Taqqu, and A. Erramilli (1996). A bibliographic guide to self-similar traffic and performance modeling for modern high-speed networks. In F. P. Kelly, S. Zachary and I. Ziedins, editors, Stochastic Networks: Theory and Applications.

[63] Zhang, L., S. Deering, D. Estin, S. Shenker, and D. Zappala (Sept. 1993). RSVP: A new resource ReSerVation Protocol. IEEE Network Mag., 7(5):8-18.

Αντιστοιχία Αγγλικών - Ελληνικών όρων

Access						Πρόσβαση

Accounting					Λογιστικά

Asynchronous Transfer Mode (ATM) 	Ασύγχρονος Τρόπος Μετάδοσης (ΑΤΜ)

Autocorrelation				Αυτοσυσχέτιση

Autocovariance				Αυτοσυνδιασπορά

Autoregressive Source			Αυτοπαλινδρομούμενη Πηγή

Available Bit Rate (ABR)			Διαθέσιμου Ψηφιακού Ρυθμού

Bandwidth					Εύρος Ζώνης

Benefit						Κέρδος

Bill						Λογαριασμός

Bound						Φράγμα

Broadband Integrated Service 		Ευρυζώνια Ψηφιακά Δίκτυα Ενοποιημένων

Digital Network (B-ISDN)			Υπηρεσιών

Buffer						Ενταμιευτής

Burstiness 					Εκρηκτικότητα

Cache						Κρυφή Μνήμη

Call Admission Control (CAC)		Έλεγχος Αποδοχής Κλήσης

Call Blocking					Φραγή Κλήσεων

Call Setup 					Εγκατάσταση Κλήσης

Cannibalization				Κανιβαλισμός

Capacity 					Χωρητικότητα

Cell						Κελί

Cell Loss Probability (CLP)			Πιθανότητα Απώλειας Κελιού (ΠΑΚ)

Central Limit Theorem			Κεντρικό Οριακό Θεώρημα

Charging					Χρέωση

Circuit Switched Network			Δίκτυο Μεταγωγής Κυκλωμάτων

Coefficient of Variation			Συντελεστής Μεταβλητότητας

Competitive Environment			Ανταγωνιστικό Περιβάλλον

Confidence Interval				Διάστημα Εμπιστοσύνης

Congestion-based-Charging			Χρέωση βάσει Συμφόρησης

Congestion Control				Έλεγχος Συμφόρησης

Connection					Σύνδεση

Connection Oriented Service			Υπηρεσία Με Σύνδεση

Connectionless Service 			Υπηρεσία Χωρίς Σύνδεση

Constant Bit Rate (CBR)			Σταθερού Ψηφιακού Ρυθμού

Contention					Ανταγωνισμός

Cost						Κόστος

Cross-Subsidization				Διασταυρωμένη Επιδότηση

Game Theory					Θεωρία Παιγνίων

Demand					Ζήτηση

Delay Jitter					Διακύμανση Καθυστέρησης

Dependence					Εξάρτηση/Συσχέτιση

Effective Bandwidth				Ισοδύναμο Εύρος Ζώνης

End User					Τελικός Χρήστης

Equilibrium Point				Σημείο Ισσοροπίας

Ergodic Stochastic Process			Εργοδική Στοχαστική Ανέλιξη

Estimator					Εκτιμητής

Fairness					Δικαιοσύνη

Feedback					Ανάδραση

Flow Control					Έλεγχος Ροής

Frame						Καρέ/Πλαίσιο

Guaranteed Service				Εγγυημένη Υπηρεσία

Heuristic Method				Ευριστική Μέθοδος

i.i.d RVs 					Ανεξάρτητες και Ισόνομες TM

Index of Dispersion				Δείκτης Διασποράς

Interactive Service				Αλληλεπιδραστική Υπηρεσία

Label Switching				Μεταγωγή Ετικετών

Large Deviations Theory			Θεωρία των Μεγάλων Αποκλίσεων

Leased Line					Μισθωμένη Γραμμή

Link						Σύνδεσμος Επικοινωνίας

Locality					Τοπικότητα

Logarithmic Moment Generating Function	Λογάριθμος Ροπογεννήτρια

Market Segmentation				Κατάτμηση Αγοράς

Minimum Cell Rate (MCR)			Ελάχιστος Ρυθμός Μεταφοράς Κελιών

Moment					Ροπή

Moment Generating Function			Ροπογεννήτρια Συνάρτηση

Monetary Unit					Νομισματική Μονάδα

Monopoly					Μονοπώλιο

Multicast					Πολλαπλή Αποστολή

Multimedia Applications			Εφαρμογές Πολυμέσων

Multiplexing Gain				Κέρδος Πολυπλεξίας

Multiservice Network				Δίκτυο Πολλαπλών Υπηρεσιών

Node						Κόμβος

Overflow					Υπερχείλιση

Overcharge					Υπερχρέωση

Packet-Switched Network			Δίκτυο Μεταγωγής Πακέτων

Point-to-multipoint Connection		Σύνδεση από σημείο-σε-πολλαπλά σημεία

Point-to-point Connection			Σύνδεση από σημείο-σε-σημείο

Price Elasticity 				Ελαστικότητα Τιμής

Pricing						Κοστολόγηση

Profit						Κέρδος

Quality-of-Service (QoS)			Ποιότητα Εξυπηρέτησης

Real Time Traffic				Κίνηση Πραγματικού Χρόνου

Recommendation				Σύσταση

Reselling					Μεταπώληση

Resource Reservation				Δέσμευση Πόρων

Resource Sharing				Κοινή Χρήση Πόρων

Revenue					Έσοδο

Routing Label					Ετικέτα Δρομολόγησης

Self-Similar Model				Αυτοπαρεμφερές Μοντέλο

Service Provider				Παροχέας Υπηρεσιών

Shadow Price					Σκιώδης Τιμή

Shared Resources				Κοινόχρηστοι Πόροι

Signaling					Σηματοδότηση

Slice						Λωρίδα

Smooth Traffic				Ομαλή Κίνηση

Social Welfare					Κοινωνική Ευημερία

Source Policing				Αστυνόμευση Πηγής

Specification					Προδιαγραφή

Standard Deviation				Τυπική Απόκλιση

Standard					Πρότυπο

Stationary Source				Στατική Πηγή

Stationary Stochastic Process			Στάσιμη Στοχαστική Ανέλιξη

Statistical Multiplexing			Στατιστική Πολυπλεξία

Stochastic Process				Στοχαστική Ανέλιξη/Διεργασία

Substitute Goods				Υποκατάστατα Αγαθά

Sufficient Statistic				Επαρκές Στατιστικό

Superposition					Επαλληλία

Supply						Προσφορά

Surplus					Πλεόνασμα

Switch						Μεταγωγέας

Switching					Μεταγωγή

Tariff						Ταρίφα/Τέλος

Telecommunication Carrier			Τηλεπικοινωνιακός Φορέας

Teleconference				Τηλεδιάσκεψη

Throughput					Διέλευση/Παροχή

Time Lag					Χρονική Απόσταση

Time Scale					Κλίμακα Χρόνου

Time-Series					Χρονοσειρά

Trace						Ίχνος

Trade-off					Αντιστάθμιση

Traffic Generator				Γεννήτρια Κίνησης

Traffic Shaping				Εξομάλυνση Κίνησης

Traffic Stream					Ροή Κίνησης

Unspecified Bit Rate (UBR)			Ακαθόριστου Ψηφιακού Ρυθμού

Usage-based-Charging			Χρέωση βάσει Χρήσης

Utilization					Χρησιμοποίηση

Variable Bit Rate (VBR)			Μεταβλητού Ψηφιακού Ρυθμού

Variance					Διασπορά

Video-on-Demand (VoD)			Video κατ’απαίτηση

Video-telephone				Εικονο-τηλέφωνο

Virtual Path					Εικονικό Μονοπάτι

Verification					Επαλήθευση

Workload					Φόρτος Εργασίας